NAME : - DISHA PRAJAPATI

# Assessment :- 3

# Network Troubleshooting

Section A: Scenario-Based Concept Application

- Explain how Deep Packet Inspection (DPI) at the Network and Transport layers of the OSI model allows a startup to isolate specific traffic bottlenecks causing slow internet speeds despite high available bandwidth.

ANS :-

DPI analyzes packet data at Network and Transport layers to identify which applications or protocols are consuming bandwidth. It helps detect traffic bottlenecks, unnecessary usage, and slow-performing services even when total bandwidth is available.

- Explain the distinct roles of DHCP and DNS in a scenario where startup devices are successfully assigned IP addresses but remain unable to resolve external web addresses for internet access.

ANS :-

DHCP automatically provides IP address, subnet mask, gateway, and DNS information to devices. DNS converts website names into IP addresses. If devices get IP addresses but websites do not open, the DNS service is likely failing.

- Explain the necessity of Inter-VLAN Routing and the specific role of static routes when a startup requires communication between two isolated departments on the same physical switch.

ANS :-

VLANs separate departments into different networks for security and management. Inter-VLAN routing allows communication between these VLANs. Static routes help routers know the correct path for sending traffic between networks.

- Explain the performance differences between ICMP and HTTP/HTTPS protocols that would cause a website to experience high latency while a ping test returns a fast, successful response.

ANS :-

ICMP ping tests use small packets and check only connectivity. HTTP/HTTPS traffic involves webpage loading, encryption, and server processing, which can increase latency even if ping responses are fast.

- Explain the conceptual logic of Firewall Access Control Lists (ACLs) and the strategic placement of a firewall to restrict unauthorized traffic between a startup’s management and general staff departments

ANS:-

ACLs control which traffic is allowed or denied based on IP addresses and protocols. A firewall is placed between management and staff networks to block unauthorized access and improve security.

Section B: Practical Application Tools: Cisco Packet Tracer, Wireshark, CLI Terminal.

- Task 1: Design a VLSM subnet scheme for 3 departments requiring 15 hosts each using $192.168.10.0/24$.

ANS:-

For 15 hosts, /27 subnet mask is required.

| Department | Network Address | Subnet Mask |
| --- | --- | --- |
| Department 1 | 192.168.10.0/27 | 255.255.255.224 |
| Department 2 | 192.168.10.32/27 | 255.255.255.224 |
| Department 3 | 192.168.10.64/27 | 255.255.255.224 |

- Task 2: Configure VLANs 10, 20, and 30 on a Cisco switch and assign physical interfaces to each.

ANS:-

Only command -

Switch(config)# vlan 10
Switch(config-vlan)# name HR

Switch(config)# vlan 20
Switch(config-vlan)# name SALES

Switch(config)# vlan 30
Switch(config-vlan)# name IT

Switch(config)# interface fa0/1
Switch(config-if)# switchport mode access
Switch(config-if)# switchport access vlan 10

- Task 3: Set up a DHCP pool on a Router/L3 switch to provide dynamic addressing for the VLAN 10 subnet.

ANS:-

Router(config)# ip dhcp pool VLAN10

Router(dhcp-config)# network 192.168.10.0 255.255.255.224

Router(dhcp-config)# default-router 192.168.10.1

Router(dhcp-config)# dns-server 8.8.8.8

- Task 4: Capture a network trace in Wireshark and identify the IP addresses in the DNS query and HTTP GET headers

ANS:-

In Wireshark:

- DNS Query shows source and destination IP addresses used for domain resolution.

- HTTP GET header shows client request to access a webpage server.

Section C: Integrated Case Study

Scenario: Tech Launch, a startup with 40 employees, is experiencing "flapping" network connectivity. Employees in the Marketing wing report that their internet drops for 30 seconds every few minutes. The Engineering wing, located on a different VLAN, remains perfectly stable. A ping test from a Marketing PC to the Core Switch shows a consistent 25% packet loss and fluctuating latency.

Questions

- Based on the symptoms, identify the two most likely OSI layers where this fault is occurring.

ANS:-

- Physical Layer

- Data Link Layer

- Outline a 4-step "Bottom-Up" troubleshooting sequence to isolate this specific issue.

ANS:-

- Check cables and switch ports

- Verify interface status and errors

- Check VLAN configuration

- Test network connectivity and packet loss

- List three CLI commands you would run on the switch to check for cable faults or broadcast storms.

ANS:-

- show interfaces

- show spanning-tree

- show mac address-table

- If the issue is a "Switching Loop," explain which protocol should have prevented this and why it might have failed.

ANS:-

STP (Spanning Tree Protocol) should prevent switching loops by blocking redundant paths. Failure may occur due to incorrect configuration or disabled STP.

- Propose a long-term configuration change to ensure an error in one department doesn't impact the rest of the office.

ANS:-

Implement proper VLAN segmentation, enable STP, and use ACLs to isolate departments so network issues in one VLAN do not affect the entire office.